Awesome Engine: done

It’s already setup and sending data to our IoT Central app.

After dealing with auth stuff, parameters and some debugging, we’ve just used a simpler solution that works very well: a very small python script (around 70 lines of code, taken from here https://github.com/Azure/iot-central-firmware/tree/master/RaspberryPi) with basic modifications to use the Grove libs. I’m attaching the file just in case it’s useful (since it’s extremely simple, no need to generate connection strings, etc.) Plus: modifying 4 lines of code, it can be used from any PC to mock the device with no other dependencies.

New simple_tumbling.py file with more comments inline. It’s embedded in this document and also stored in the same location as this document.

Setup simplified basic instructions:

- The script can be stored in any place, and it’s not automatically fired on startup. In our environment, we manually (for the moment) startup every time with a console and the command python simple_tumbling.py. It gives some output to the console.

- The script uses a python library used to send data form Raspberry Pi to IoT Central. The library is named iotc and needs to be installed using pip install iotc. It’s documented here: https://github.com/Azure/iot-central-firmware/tree/master/RaspberryPi

- Also, once iotc is imported, you need to get 3 values from IoT Central device and paste in the script. In IoT Central, go to device page, click in connect on the top right corner, copy-paste these parameters.

- The script also uses a library named grove_dht_pro_once, used for reading temperature and humidity from sensors. Nothing is needed, just make sure that the path where grove sdks are installed is in the path (it should be already)

- The vibration is taken from a file. For some reason, there’s another process launched automatically at startup that starts reading vibration from the sensor and writing into a file. The python script just reads the text file to get the value. The file path is “/ramdisk/vibration.value”.

- The startup stuff (can be used to change the device small screen text, as in the first picture of this document), is in a file named init.d stored at /etc/init.d. Although this file executes stuff in startup, I’ve been told that this is not the right place to launch our python script. I don’t know exactly why (not an expert!), so we used this to change the small green screen, but nothing more.